1. Project Description

Integrated Smart POS Network & Infrastructure Report

A smart POS network connecting multiple branches to a central server, analyzing sales, detecting suspicious transactions, and helping managers monitor operations securely.

2. Network Topology & Addressing

Figure 1: GNS3 Network Topology for the Smart POS Project

| Device | Interface | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| Router R4 | f0/0 (WAN) | 10.0.0.1 | 255.255.255.252 | - |
| Router R4 | f0/1 (LAN) | 192.168.1.1 | 255.255.255.0 | - |
| Router R5 | f0/0 (WAN) | 10.0.0.2 | 255.255.255.252 | - |
| Router R5 | f0/1 (LAN) | 192.168.10.1 | 255.255.255.0 | - |
| Cashier Desktop | eth0 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| Admin Server | eth0 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |

3. Router Configurations

# Router R4 (Branch Gateway)

enable

configure terminal interface f0/0

ip address 10.0.0.1 255.255.255.252

no shutdown exit

interface f0/1

ip address 192.168.1.1 255.255.255.0

no shutdown exit

ip route 192.168.10.0 255.255.255.0 10.0.0.2

end write

Router R5 (HQ/Admin Gateway)

enable

configure terminal interface f0/0

ip address 10.0.0.2 255.255.255.252

no shutdown exit

interface f0/1

ip address 192.168.10.1 255.255.255.0

no shutdown exit

ip route 192.168.1.0 255.255.255.0 10.0.0.1

end write

4. System Deployment (Ubuntu Admin Server)

sudo apt update && sudo apt upgrade

sudo apt install git python3 python3-pip python3-venv -y python3 -m venv venv

source venv/bin/activate

pip install uvicorn fastapi PyJWT

uvicorn main:app --host 0.0.0.0 --port 8000 --reload

Client Web Access: http://192.168.10.10:8000

5. (Bonus) Wireless Infrastructure & Hardware

Integration

# Operating System Configuration (OS Environment)

Initially, the project utilized a standard OpenWrt Generic Image. However, we encountered significant technical bottlenecks when attempting to install the Tailscale package, including dependency conflicts and storage constraints.

Technical Solution: We pivoted to a Custom Image Building approach using the OpenWrt Image Builder, pre-installing tailscale, tailscaled, and mt7601u drivers.

Figure 2: Successful Tailscale Activation

# Physical Hardware Integration (USB Passthrough)

To transition to a realistic wireless setup, we integrated a MediaTek MT7601U USB Adapter with the GNS3 emulator via a multi-layer mapping process (VMware USB 2.0 Controller and Direct Passthrough).

Figure 3: MediaTek MT7601U USB Adapter (Model UWDED)

# Verification & Implementation Challenges

The system successfully returned the hardware signature: Bus 001 Device 002: ID 148f:7601 MediaTek 802.11n WLAN, confirming successful passthrough.

AP Mode Limitation: We encountered a limitation where the driver remained in "Managed Mode" (Receiver) and refused to switch to "Master Mode" (Broadcaster).

Engineering Decision: We adopted a Wired Solution to ensure maximum data reliability and zero latency for real-time processing requirements.

Smart POS Project Report | GNS3 Infrastructure Documentation | 2026